Supplementary material.

Table S1. MS/MS parameters obtained for a standard solution of isoflavones at 1000 mgL-1. Optimized conditions: capillary voltage: 3500 V, nebulizer pressure: 35 psi, drying gas flow rate: 12 L min-1, temperature of the gas: 350ºC. ESI-MS spectra were acquired in positive-ion MRM mode.

| Analtyte | Precursor ion (m/z) | Product ions (m/z) | Fragmentor (V) | Collision energy (V) |
| --- | --- | --- | --- | --- |
| Daidzin | 417.1 | 255.1* | 110 | 12 |
| Glycitin | 447.1 | 285.1*/270.1 | 90 | 8/40 |
| Genistin | 433.1 | 271.1* | 90 | 16 |
| Daidzein | 255.1 | 137*/199.1/91 | 150 | 28/24/40 |
| Glycitein | 285.1 | 270*/242 | 150 | 24/32 |
| Genistein | 271.1 | 153*/91.1 | 150 | 28/40 |
| Formononetin | 269.1 | 197.1*/253 | 150 | 40/28 |
| Biochanin A | 285.1 | 152*/213 | 150 | 24/40 |

*These product ions were used for the quantification; the rest of the product ions were used for confirmatory analysis

Table S2. Box-Behnken experimental design matrix.  The letters A to E correspond to the factors in Table S2.

| Experiment | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -1 | -1 |
| 2 | 0 | 0 | 0 | 1 | -1 |
| 3 | 0 | 0 | 0 | -1 | 1 |
| 4 | 0 | 0 | 0 | 1 | 1 |
| 5 | 0 | -1 | -1 | 0 | 0 |
| 6 | 0 | 1 | -1 | 0 | 0 |
| 7 | 0 | -1 | 1 | 0 | 0 |
| 8 | 0 | 1 | 1 | 0 | 0 |
| 9 | -1 | 0 | 0 | -1 | 0 |
| 10 | 1 | 0 | 0 | -1 | 0 |
| 11 | -1 | 0 | 0 | 1 | 0 |
| 12 | 1 | 0 | 0 | 1 | 0 |
| 13 | 0 | 0 | -1 | 0 | -1 |
| 14 | 0 | 0 | 1 | 0 | -1 |
| 15 | 0 | 0 | -1 | 0 | 1 |
| 16 | 0 | 0 | 1 | 0 | 1 |
| 17 | -1 | -1 | 0 | 0 | 0 |
| 18 | 1 | -1 | 0 | 0 | 0 |
| 19 | -1 | 1 | 0 | 0 | 0 |
| 20 | 1 | 1 | 0 | 0 | 0 |
| 21 | 0 | 0 | -1 | -1 | 0 |
| 22 | 0 | 0 | 1 | -1 | 0 |
| 23 | 0 | 0 | -1 | 1 | 0 |
| 24 | 0 | 0 | 1 | 1 | 0 |
| 25 | 0 | -1 | 0 | 0 | -1 |
| 26 | 0 | 1 | 0 | 0 | -1 |
| 27 | 0 | -1 | 0 | 0 | 1 |
| 28 | 0 | 1 | 0 | 0 | 1 |
| 29 | -1 | 0 | -1 | 0 | 0 |
| 30 | 1 | 0 | -1 | 0 | 0 |
| 31 | -1 | 0 | 1 | 0 | 0 |
| 32 | 1 | 0 | 1 | 0 | 0 |
| 33 | -1 | 0 | 0 | 0 | -1 |
| 34 | 1 | 0 | 0 | 0 | -1 |
| 35 | -1 | 0 | 0 | 0 | 1 |
| 36 | 1 | 0 | 0 | 0 | 1 |
| 37 | 0 | -1 | 0 | -1 | 0 |
| 38 | 0 | 1 | 0 | -1 | 0 |
| 39 | 0 | -1 | 0 | 1 | 0 |
| 40 | 0 | 1 | 0 | 1 | 0 |
| 41 | 0 | 0 | 0 | 0 | 0 |
| 42 | 0 | 0 | 0 | 0 | 0 |
| 43 | 0 | 0 | 0 | 0 | 0 |

Table S3: Coefficients of the factors in the selected model for each analyte together with their corresponding p-value.

| Analyte |  | Factors in the selected model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | A2 | B2 | C2 | D2 | E2 |
| Daidzin | Coefficient | 16.05 | -4.69 | 6.49 | -11.59 | 0.75 |  |  |  |  |  |
|  | p-value | 0.0001 | 0.2167 | 0.0908 | 0.0037 | 0.8411 |  |  |  |  |  |
| Glycitin | Coefficient | 10.61 | -2.90 | 5.11 | -0.93 | 3.31 |  |  |  |  |  |
|  | p-value | 0.0016 | 0.3597 | 0.1107 | 0.7686 | 0.2969 |  |  |  |  |  |
| Genistin | Coefficient | 3.96 | -0.46 | 4.47 | 5.38 | -1.95 | -15.75 | -8.10 | -10.28 | 3.18 | -10.42 |
|  | p-value | 0.1321 | 0.8589 | 0.0903 | 0.0436 | 0.4507 | 0.0016 | 0.0851 | 0.0311 | 0.4904 | 0.0290 |
| Daidzein | Coefficient | 4.51 | -1.33 | 7.09 | 4.56 | 1.75 |  |  |  |  |  |
|  | p-value | 0.1150 | 0.6374 | 0.0155 | 0.1112 | 0.5357 |  |  |  |  |  |
| Glycitein | Coefficient | 5.29 | -1.18 | 5.65 | 5.75 | 2.25 |  |  |  |  |  |
|  | p-value | 0.0476 | 0.6497 | 0.0351 | 0.0321 | 0.3890 |  |  |  |  |  |
| Genistein | Coefficient | - | - | - | - | - | - | - | - | - | - |
|  | p-value | - | - | - | - | - | - | - | - | - | - |
| Formononetin | Coefficient | 2.35 | -0.92 | 5.98 | 7.45 | 1.10 |  |  |  |  |  |
|  | p-value | 0.3957 | 0.7372 | 0.0348 | 0.0097 | 0.6901 |  |  |  |  |  |
| Biochanin A | Coefficient | -0.81 | 0.43 | 4.79 | 11.12 | 1.21 |  |  |  |  |  |
|  | p-value | 0.7912 | 0.8893 | 0.1229 | 0.0008 | 0.6913 |  |  |  |  |  |

A: Amount of sorbent (mg); B: extraction time (min); C: Desorption time (min); D: Volume of the desorption solvent (mL); E: pH of the desorption solvent

Figure S1. Influence of desorption solvent: a) MeOH, b) MeCN, c) MeCN:MeOH 1:2 (v/v).